Master Thesis

Design and Development of a MATLAB to Python GUI for Flexible Biogas Production; A Case Study on Biogas Production Management.

Submitted by:

Sanju Pallissery Babu

(Matriculation number: 03745203)

| Date of Submission: | 02.04.2025 |
| --- | --- |
| Examiner: | Prof. Dr. Matthias Gaderer (Technical University of Munich, Straubing) |
| Supervisors: | Lingga Aksara Putra (Technical University of Munich, Straubing) |

Affidavit

I hereby certify that this master's thesis was written without the help of third parties and only with the sources and aids indicated. All passages used have been identified. This thesis has not yet been submitted in the same or a similar form to any examination authority.

Sanju Pallissery Babu

Straubing, 01.04.2025

Abstract

This thesis focuses on the development of an accessible control system for flexible biogas production, converting MATLAB-based algorithms to Python to improve affordability and usability in response to the rising demand for biogas plants to operate flexibly inside renewable energy networks. The study used the Grey-Box method for system modeling, which combines theoretical structure with data-driven parameter identification. Six modeling approaches were evaluated using the coefficient of determination (R²), with the time percentage method being the most effective for the upward production model. This model was implemented with a state space controller, which demonstrated stable, non-oscillating behavior and efficient setpoint tracking in simulations, while the Python implementation has an intuitive graphical interface that automatically generates practical feeding schedules with precise substrate quantities at specified intervals. Simulation results confirm that the controller successfully manages transitions from initial biogas production rates to higher setpoints via systematic substrate feeding adjustments, laying the groundwork for improving both the economic viability of biogas plants and their contribution to grid stability in renewable energy systems, while also providing plant operators with precise control without the need for extensive theoretical knowledge.

Contents

1	Introduction	1

2	Literature review	3

2.1	Matlab	6

2.2	Python	8

2.2.1	Features of Python	8

3	Materials	1

4	Methodology	1

4.1	Approach to Conversion	1

4.2	Load Data Tab	2

4.2.1	Overview	2

4.3	Preprocessing Tab	5

4.3.1	Preprocessing of Biogas Production Data	5

4.3.2	Visualization	6

4.3.3	GUI Interaction and Control	7

4.4	Model Estimation Tab	7

4.4.1	PT1 - Approximation	8

4.4.2	Time Percentage Method	11

4.4.3	Sum of Time Constants Method	15

4.4.4	Turning Tangent Method	18

4.4.5	PT1-Estimator	22

4.4.6	PT2-Estimator	24

4.5	Control system	27

The feedback control system implements a state space controller for an upward model of biogas production. The controller implements a deadbeat control strategy using Ackermann’s formula to place poles at the origin, thereby guaranteeing a rapid system response. The feedback control system for the upward model follows the design principles illustrated in (Fig.), which involve a prefilter F and state feedback gain K working together to accomplish the desired closed-loop behavior.	27

4.5.1	System Identification and Modeling	27

4.5.2	State Space Representation	27

4.5.3	Controller Design	27

4.5.4	Prefilter Design	28

4.5.5	Simulation Framework	28

5	Result and Discussion	29

6	References	29

List of Figures

Figure 1:	Donkey Rocket-Early stage	Error! Bookmark not defined.

Figure 2:	Upper Column part of a rectification column with feed-in point for entrainer	Error! Bookmark not defined.

Figure 3:	Example Excel-graphic; Fermentation process of sweet sorghum pressed juice in a 25-liter batch at 35 °C	Error! Bookmark not defined.

Figure 4:	Setting the style sheets for the formula editor	Error! Bookmark not defined.

List of Tables

Table 1:	Example table with particularly long labeling to demonstrate the settings for the table labeling	Error! Bookmark not defined.

Table 2:	Styles, font sizes and spacing	Error! Bookmark not defined.

Table 3:	Formatting without style sheets	Error! Bookmark not defined.

Table 4:	Indents and tab stops	Error! Bookmark not defined.

Nomenclature

| Symbol | Description | Unit |
| --- | --- | --- |
| Latin Symbol |  |  |
| A | System matrix |  |
| B | Input matrix |  |
| C | Output matrix |  |
| F | Prefilter gain |  |
| G(s) | Continuous transfer function |  |
| G(z) | Discrete transfer function |  |
| K | System gain |  |
| T | Time constant | (Seconds) |
| Ta | Sampling time | (Seconds) |
| Tb | Second time constant in PT2 model | (Seconds) |
| Tg | Rise time | (Seconds) |
| Tu | Delay time | (Seconds) |
| a, b, c, d, e, f | Coefficients in discrete transfer function |  |
| n | System order |  |
| R2 | Coefficient of determination |  |
| s | Laplace variable |  |
| t | Time | (Seconds) |
| u(t) | System input at time t | (th) |
| x | State vector |  |
| y(t) | System output at time t | (m3h) |
| Indices |  |  |
| T63 | Time when output reaches 63% of final value | (Seconds) |
| T72% | Time when output reaches 72% of final value | (Seconds) |
| a1, a2 | Parameters of second order difference equation |  |
| b1, b2 | Parameters of second order difference equation |  |
| Gd | Discrete transfer function |  |

Abbreviations

| ADM1 | Anaerobic Digestion Model No.1 |
| --- | --- |
| AIC | Akaike Information Criterion |
| CHP | Combined Heat and Power |
| CSV | Comma-Separated Values |
| GUI | Graphical User Interface |
| MATLAB | Matrix Laboratory |
| MAE | Mean Absolute Error |
| MPC | Model Predictive Control |
| PI | Proportional-Integral (controller) |
| PID | Proportional-Integral-Derivative (controller) |
| PLC | Programmable Logic Controller |
| PSO | Particle Swarm Optimization |
| PT1 | First-order transfer function model |
| PT2 | Second-order transfer function model |
| SSE | Sum of Squares Error |
| TCL | Tool Command Language |
| ZOH | Zero-Order Hold |

# Introduction

Climate change has emerged as one of the most pressing global challenges of the 20th century [1]. As a significant factor to this crisis, fossil fuels, including coal, oil, and natural gas, have long served as the primary energy sources, accounting for nearly 80% of global energy consumption [2]. However, their extensive use has led to severe environmental consequences, including greenhouse gas emissions, pollution, and resource depletion [3]. Additionally, the increasing energy demand from industries is accelerating the depletion of fossil fuel reserves [4]. In response to these challenges, renewable energy sources are gaining prominence, with biogas playing a crucial role in reducing carbon dioxide emissions and promoting sustainable energy generation [5].

Biogas is produced through the anaerobic decomposition of organic materials by microorganisms. Common feedstocks for biogas production include agricultural manure, sewage, municipal waste, green waste, plant material, and other biodegradable substances [6]. Renewable energy sources such as solar, wind, hydroelectric, biomass, and geothermal biogas hold a unique position due to their ability to provide a clean energy alternative while simultaneously offering benefits such as waste management and the production of high-quality organic fertilizer [7].

A biogas plant is a complex system comprising multiple components designed to efficiently convert biomass into biogas. Key elements include substrate processing, fermentation, gas storage and treatment, digestate storage, and power generation [8]. The process begins with substrate processing, where organic waste such as animal manure, agricultural residues, and silage are prepared to maximize gas yield and prevent operational issues. Fermentation, the core stage of biogas production, occurs in an oxygen-free (anaerobic) environment, where microorganisms break down organic materials, ultimately producing methane-rich biogas. Once produced, the biogas undergoes storage and purification, followed by combustion in combined heat and power (CHP) plants to generate electricity and thermal energy [9].

Amid rising electricity and gas prices and the environmental impact of fossil fuels, the need for cost-effective and scalable biogas solutions has become increasingly important [10]. Ensuring the economic viability of biogas plants requires the development of new, sustainable business models that enhance energy production and allow for flexible biogas generation. One key aspect of optimizing biogas production is the implementation of mathematical biogas plant models and adaptive feedback control systems [11].

To achieve this, a MATLAB-based graphical user interface (GUI) has been developed to facilitate the monitoring and optimization of biogas production. The current GUI is based on a discrete feedback control method verified at the Grub biogas plant in Germany. It employs Grey-Box modeling techniques with PT1 and second-order system approximations and utilizes PI and state-space controllers to dynamically regulate biogas production in response to fluctuations in substrate input [11]. However, MATLAB is a proprietary software with high licensing costs, making it less accessible for small-scale plant owners and researchers [12]. This limitation necessitates the transition to an open-source, cost-effective alternative.

This study focuses on the design and development of a Python-based GUI that replicates the functionality of the existing MATLAB interface while ensuring affordability and ease of use. By leveraging open-source tools and programming frameworks, the proposed Python system aims to provide efficient, scalable, easy-to-maintain multiple versions of shared libraries, more compact and readable than MATLAB code, and a user-friendly platform for biogas plant operators [12]. The transition from MATLAB to Python reduces software costs and increases accessibility, enabling more widespread adoption of biogas optimization technology [13].

Through this research, a case study on biogas production management will be conducted to validate the effectiveness of the newly developed Python-based GUI. The study will assess its ability to streamline operations, enhance control mechanisms, and maximize biogas yield while maintaining reliability and usability for plant operators.

## Thesis Structure

A systematic development strategy was employed to shift from a MATLAB-based GUI to a Python-based platform for optimizing biogas output. Figure 1.1: Thesis Structure depicts this procedure, commencing with the analysis of the MATLAB GUI, defining its fundamental functionalities, and selecting suitable Python libraries. The subsequent phase entails reconfiguring the data process, utilizing Grey-Box modeling methodologies, and executing feedback control mechanisms. A multi-tabbed Python GUI has been designed to facilitate data loading, preprocessing, model estimate, control simulation, and feeding schedule development. This systematic methodology guarantees a scalable, user-centric, and economical instrument for real-time biogas facility administration.

Figure 1.1: Thesis Structure

Research Question:

- What are the computational challenges and algorithmic modifications required when effectively replacing a MATLAB-based feedback control system for flexible biogas production with a Python-based platform to enhance accessibility, cost-effectiveness, and user experience while maintaining system accuracy and control efficiency?

# Literature review

## Biogas; A suitable solution for energy and climate challenges

One of the main causes of climate change is the continued significant reliance on fossil fuels brought on by the world's steadily rising energy consumption [14]. There is broad scientific agreement that the earth is warming at an alarming rate due to greenhouse gas emissions from burning coal, oil, and natural gas [15]. Observable phenomena including extreme weather events, rising sea levels, and ecological damage are becoming more and more indicative of climate changes. A fundamental shift away from carbon-intensive energy sources and toward renewable alternatives is necessary to meet this issue [16]. Because it uses organic waste products to produce clean energy, biogas is a particularly useful alternative in this situation. Biogas production from food waste, agricultural wastes, and other biodegradable materials has two advantages for the environment: it prevents methane emissions that would otherwise happen during natural decomposition while also producing renewable energy [7]. According to research, any successful climate mitigation plan must include the broad use of biogas and other renewable energy technology.

### Biogas Production and Anaerobic Digestion

Biogas is produced through the anaerobic digestion of biodegradable organic materials such as animal manure, agricultural waste, sewage sludge, and municipal organic waste [17]. This process involves microbial decomposition in an oxygen-free environment, resulting in the production of methane (CH₄) and carbon dioxide (CO₂). Anaerobic digestion is a complex biological process that requires stable conditions, including temperature control and substrate balance. Due to fluctuations in substrate composition, microbial activity, and environmental conditions, the manual operation of biogas plants often leads to inefficiencies in gas production [18]. Figure 2.1 illustrates the entire process flow of a typical biogas production system, from substarte handling through digestion to the generation of electricity and heat using combined heat and power (CHP) units as well as the production of fetilizer from the resultant digestate.

Figure 2.1: Biogas production process and energy generation flow

### Modeling and control approaches in biogas systems

To improve efficiency, system modeling, and simulation techniques are used to predict biogas yields and optimize plant operation. Different levels of model approaches to biogas production are categorized as White-box, Grey-box, and Black box or bottom-up and top-down approaches [19]. These biogas models can provide dynamic information on the anaerobic digestion process, which can be used to forecast biogas yields or optimize the process.

### White box, Black box and Grey box Method

White-box models are physics-based models that describe systems using mathematical equations based on fundamental laws, such as mass balance and reaction kinetics. These models are categorized as linear or nonlinear differential equations that predict system behaviour deterministically [20].

White-box models are difficult to develop and implement because of their complexity, which necessitates the design and implementation of all influencing parameters [21]. The widely used White-box model is Anaerobic Digestion Model No.1(ADM1). ADM1 includes biochemical reactions and physicochemical reactions to simulate biogas production[21]. ADM1 is effective in identifying process inhibitors such as ammonia toxicity and pH imbalances. However, it was originally developed for wastewater treatment plants, and its direct application to agricultural biogas plants is limited due to variations in substrate composition [22]. Agricultural substrates contain a mix of proteins, fats, fibres, and carbohydrates, which require model modifications for accurate biogas yield predictions [23].

Moreover, an essential part of the model validation is the analysis of the residual errors, which refer to the difference between the actual and predicted value in a statistical or machine learning model. The white-box model often struggles to produce completely random errors (uncorrelated); instead, its error shows a pattern. This happens because, while White-Box models are designed for accuracy, they tend to be either too complex or too restrictive in their assumptions. Even when they work well on paper, they often miss important real-world factors like how well a digester is mixed, which plays a big role in biogas production [11]. Hence, white-box models require more advanced differential equations, such as stochastic differential equations [20]. to account for the natural uncertainties in the system.

Black-box models, like machine learning and deep learning, can work with complex and nonlinear data patterns [24]. Black-box models use statistical methods, experiments, or real-world measurements to find relationships between input and output variables. While Black-box models offer high adaptability and predictive power, they also have certain limitations., like they requires large amount of training data, lack interpretability, making its difficult to understand why certain predictions are made and they fails in unseen scenarios [25].

Grey-Box model is a hybrid model, it is the combined form of both White-box and Black-box models. Grey-box have both physical and empirical significance [19]. Grey-box has the capability to handle complex systems more efficiently than the white-box and black-box models. It requires less data and have improved generalization and adaptability. This model is attempted, the algorithms were only evaluated on a simulation. Furthermore, the idea has only been investigated for one substrate, but the effect of various substrate interactions on the outcomes is still unknown [11].

### Feedback control system

Once a system model is in place, the next crucial step is designing a feedback control mechanism to ensure smooth and efficient operation. Traditional controllers, like Proportional-Integral (PI) and Proportional-Integral-Derivative (PID) controllers, are widely used because they are both simple and effective at maintaining stable conditions [26]. These controllers work particularly well in systems with minimal disturbances and steady setpoints, so they are popular in industrial settings such as biogas plants where reliability and consistency are key [27].

Traditional controllers might not work well in more complicated, multivariable systems where constraints require careful management and setpoints are subject to frequent changes. Model Predictive Control (MPC) and other sophisticated control techniques are useful in this situation. Utilizing a dynamic model, MPC forecasts future system behavior and modifies control actions accordingly [28]. MPC is particularly useful in settings requiring frequent setpoint changes and stringent constraint management because it can manage several variables and constraints simultaneously. In demanding applications, MPC clearly outperforms conventional PI controllers thanks to its predictive capability.

Because of variations in substrate inputs, gas production rates, and environmental factors, operating conditions in biogas plants are rarely constant [29]. PI controllers may find it difficult to adjust to these fluctuations, even though they can efficiently maintain steady-state operations [30]. Model Predictive Control (MPC) is helpful in this situation. Through constant real-time data analysis and predictive adjustments, MPC enhances system performance more dynamically [31]. Nevertheless, there are difficulties in putting MPC into practice despite its benefits. It is more difficult to set up than traditional controllers because it needs a well-calibrated system model and more processing power [32].

Therefore the choice between MPC and traditional PI control ultimately depends on the complexity of the system, its performance needs, and how practical it is to implement [33]. While PI controllers work well for maintaining stability, MPC offers greater flexibility in dynamic environments. To further improve prediction accuracy and adapt more effectively to changing substrate compositions, future research should explore integrating grey-box modeling with MPC. Additionally, real-world testing is essential going beyond simulations will help verify the reliability and practical benefits of these advanced control strategies in biogas production systems [34].

A feedback control system is under development, especially for the upward model in the Python GUI implementation. The controller design seeks to orient all closed-loop poles at the origin [35]. But given a second-order difference equation, the upward model shows more complexity than the descending one [36]. This complexity makes the root locus approach less applicable.

While creating a feedback control system with the root locus approach is still possible, even with a PID controller relocating all closed loop poles to the origin proves impossible.  As such, the upward model will be implemented using a state space controller [37]. The major benefit of the state space approach is that it lets direct manipulation of the internal state variables of the system rather than only considers input-output interactions. This capacity is especially important in resolving the complicated dynamics inherent in biogas generation as production rises [38].

State space controllers provide significant benefits for the management of intricate biogas systems, especially for upward models that require enhanced output to satisfy fluctuating energy demands. Unlike traditional PI controllers, state space controllers utilise a mathematical representation of the system through state, input, and output variables in matrix form, facilitating more accurate modelling of multivariable dynamics [39]. In biogas applications, state space controllers exhibit higher efficiency in managing the nonlinear traits of the anaerobic digestion process, yielding enhanced response times and superior setpoint tracking compared to conventional control methods [40]. State space controllers enable the positioning of all eigenvalues of the closed-loop system at specified locations, hence facilitating optimal dynamic performance using pole placement methods. This technique has been empirically proven in biogas applications, demonstrating considerable reductions in settling time compared to PID controllers when adjusting to changes in substrate composition [41].

### Evaluation

To analyze the optimal model by their validity and predictive capacity, several statistical metrics have been used. The Akaike Information Criterion (AIC) functions as a primary metric for model selection in this work, providing a fair evaluation of model fit while imposing penalties for excessive complexity. Akaike defines AIC as AIC=2K-2In(L), where K denotes the number of parameters and L signifies the model's greatest probability [42]. This formulation offers a quantitative assessment that incentivises goodness of fit while deterring overfitting. AIC has proven to be superior in selecting parsimonious models for biogas production modeling, effectively capturing complicated biological processes without excessive parameterisation.

The coefficient of determination (R²) is commonly employed in model evaluation, although its use in nonlinear systems necessitates cautious interpretation. R² numbers may be deceptive in nonlinear scenarios, such as biogas production modelling [43]. Their research underscored cases where models with elevated R² values yielded inferior predictive performance compared to those with diminished values, advising caution about excessive dependence on this statistic alone. In biogas process modelling, R² should be regarded as a supplementary metric rather than a conclusive selection criterion [44].

The Sum of Squares Error (SSE) provides an additional helpful perspective by quantifying the overall divergence between observed and model-predicted values. SSE alone fails to consider model complexity, potentially promoting overparameterized models that excel on training data yet generalise inadequately [45]. In biogas production, where process variability are prevalent, the Mean Absolute Error (MAE) serves as a more intuitive and resilient metric of model accuracy, exhibiting reduced sensitivity to outliers compared to the Sum of Squared Errors (SSE) [46].

The ultimate model selection procedure must incorporate various factors, acknowledging that no singular indicator encompasses all dimensions of model performance. Integrating information-theoretic methodologies (e.g., AIC) with empirical error measurements (such as MAE) and validation strategies establishes a robust framework for model selection [47]. This balanced methodology is especially appropriate for biogas production modelling, where both theoretical accuracy and practical usability are critical factors.

## Graphical User Interface (GUI)

A Graphical User Interface (GUI) is a visual framework enabling user interaction with computer systems via graphical elements, including buttons, menus, sliders, icons, and input fields [48]. In contrast to text-based command-line interfaces that necessitate the learning of individual instructions, graphical user interfaces (GUIs) offer an intuitive and user-friendly environment that facilitates complex operations [49]. In technical domains like engineering, industrial automation, and scientific computing, GUIs are essential for rendering complex systems accessible to users with diverse technical proficiencies [50]. By concealing backend activities and algorithms behind graphical elements, GUIs improve usability, diminish the probability of input errors, and facilitate efficient real-time decision-making.

In software development, numerous platforms exist for creating and implementing GUIs, each providing distinct functionalities and tailored to specific domains. MATLAB and Python are two extensively utilized platforms, particularly in control system design, simulation, and data-driven applications [51]. MATLAB, a proprietary high-level programming language and environment created by MathWorks, is widely utilized in both academia and industry for its inherent functions in modeling, control design, numerical analysis, and its integrated graphical user interface creation tools, including GUIDE and App Designer [52, 53]. These tools enable users to create graphical applications that directly interact with MATLAB's computational engine, facilitating smooth simulation and real-time visualization of system dynamics.

A MATLAB-based GUI was created to optimize biogas production by integrating feedback control methods and system modeling for flexible substrate feeding. This interface enabled users, particularly plant operators, to input various feeding plans and monitor their effects on biogas production over time. The GUI enabled the simulation of control techniques, including state-space and PI controllers, allowing for the precise adjustment of system parameters prior to application in an actual biogas plant. The technology facilitated data-driven decision-making and enhanced operational efficiency in biogas production systems by showing model behavior and system response in real-time.

Nonetheless, despite its formidable capabilities, MATLAB presents some constraints, chiefly its exorbitant licensing fees and proprietary architecture . These limitations diminish accessibility for small-scale biogas facilities, independent researchers, and organizations with constrained budgets. Moreover, MATLAB's proprietary environment constrains the scalability and customisation of programs, especially in scenarios necessitating interaction with open data platforms or web-based services [54]. Consequently, the necessity for a more adaptable, transparent, and economical alternative has become increasingly evident.

This thesis aims to convert the current MATLAB-based GUI to a Python-based platform to tackle these problems. Python is a prevalent, open-source programming language that provides a comprehensive framework for scientific computing, data analysis, and graphical user interface creation. Python can emulate the computational functionalities of MATLAB with the use of modules like as NumPy, SciPy, Matplotlib, and the Control Systems Library [55]. Additionally, GUI development frameworks such as Tkinter, PyQt, and Kivy facilitate the design of interactive and visually cohesive interfaces appropriate for engineering applications [56]. The open-source nature of Python removes license expenses and offers flexibility for customization, scalability, and integration with contemporary data infrastructure. This renders it especially appropriate for practical application across many scales of biogas production systems, ranging from small farms to huge industrial facilities.

This research seeks to construct a Python-based GUI that replicates the functionality of the MATLAB version, so creating a more accessible, cost-effective, and sustainable tool for biogas plant management. The shift guarantees the retention of essential control system characteristics while facilitating new opportunities for development, community-driven improvements, and wider acceptance in the renewable energy sector.  This transition immediately facilitates the study objective of enhancing operational flexibility and control efficiency via economical and open-source technologies.

### MATLAB for GUI Development

MATLAB has become a prevalent computing environment in academia and research owing to its versatility, intuitive interface, and strong analytical skills.  It facilitates a wide array of tasks, encompassing algorithm development, numerical analysis, data visualization, and system simulation, rendering it very appealing for scientific and engineering applications.  The amalgamation of numerical and symbolic computation with advanced graphical visualization tools allows users to develop, simulate, and analyze intricate models interactively.  Numerous scholars have observed that MATLAB is among the most widely utilized Computer Algebra System (CAS) tools for mathematical exploration, problem-solving, and model visualization [48].

MATLAB's vast array of specialized toolboxes is one of its main advantages.  These toolboxes are intended to tackle particular computational tasks, including optimization, control systems, and signal processing.  In the realm of biogas plant optimization, MATLAB offers specialized toolboxes for control and simulation, enabling researchers to model system dynamics and execute feedback control schemes.  For example, MATLAB's optimization toolbox is constructed on a modular structure that facilitates the implementation of diverse optimization methods.  Users may expand or change these algorithms through a well defined interface, hence augmenting the toolbox's adaptability to various control settings [49], [50].

Commonly utilized optimization algorithms in MATLAB include the Genetic Algorithm, Direct Search Toolbox, Particle Swarm Optimization (PSO), Differential Evolution, Covariance Matrix Adaptation Evolution Strategy (CMA-ES), and Enhanced Evolution Strategy employing Stochastic Ranking.  These strategies have been utilized in various control system investigations and are esteemed for their efficacy in addressing nonlinear, non-convex, and intricate optimization challenges [51]–[55].

Notwithstanding its advantages, MATLAB possesses significant restrictions.  A significant disadvantage is its proprietary characteristic.  The techniques employed in numerous built-in functions of MATLAB are proprietary, preventing users from accessing or altering the underlying code.  This absence of transparency necessitates that consumers fully trust the accuracy of the implementations.  Moreover, the expense of MATLAB licenses is excessively high for several independent researchers and smaller universities, constraining accessibility.  Code portability is a significant issue; programs created in MATLAB typically want the identical version of the MATLAB Component Runtime (MCR) to operate on different systems.  Due to MATLAB's biannual version upgrades, ensuring compatibility can be burdensome.  Finally, MATLAB’s closed ecosystem inhibits third-party developers from enhancing the platform’s fundamental capabilities, thereby constraining innovation beyond MathWorks’ toolchain.

Nevertheless, MATLAB continues to be an industry-standard instrument for GUI development in engineering, owing to its established ecosystem, integrated simulation environment, and the simplicity with which intricate control techniques may be executed and viewed.

### Python for GUI Development

Python has become one of the most extensively utilized programming languages in scientific research, software development, and industrial applications.  Its success arises from a confluence of simplicity, readability, and an extensive ecosystem of open-source resources.  In recent years, Python has emerged as a favored alternative to proprietary platforms such as MATLAB for various tasks in data analysis, control system simulation, machine learning, and GUI creation.  In contrast to MATLAB, Python is freely accessible and offers cross-platform interoperability, rendering it an appealing choice for both academic and commercial endeavors.

Python provides various libraries for GUI creation that offer differing levels of capability and user interface control.  Commonly utilized frameworks include Tkinter, included with Python and appropriate for basic GUI applications; PyQt and PySide, which provide advanced features and contemporary GUI design principles; and Kivy, which excels in multi-touch applications and cross-platform deployment.  These libraries empower developers to construct intuitive, interactive graphical interfaces that can connect effortlessly with Python's numerical and data processing capabilities.

In the realm of biogas plant modeling and control, Python serves as a formidable alternative to MATLAB, owing to its robust scientific computing libraries, including NumPy, SciPy, and Matplotlib, alongside specialized packages such as the Control Systems Library and SymPy for symbolic mathematics.  These libraries offer functionality comparable to other MATLAB toolboxes, facilitating the creation of dynamic models, execution of feedback control systems, and observation of system performance.  Furthermore, Python's adaptability enables users to customize algorithms and visualizations to meet individual project requirements, providing a greater level of personalization than MATLAB's proprietary framework.

A significant benefit of Python is its open-source characteristic, which fosters community-driven development and swift innovation.  Developers can view and alter the source code of libraries and frameworks, guaranteeing transparency and flexibility.  This is especially advantageous in research environments, where reproducibility and extensibility are essential.  Moreover, Python facilitates interaction with contemporary technologies, including web-based dashboards, cloud computing platforms, and machine learning pipelines—attributes that are progressively pertinent in the advancement of scalable and intelligent energy systems.

This thesis selects Python as the platform for recreating the MATLAB-based GUI because of its accessibility, versatility, and cost-effectiveness.  By emulating the essential functionalities of the MATLAB GUI—such as data loading, preprocessing, model estimate, control simulation, and feeding schedule generation—within a Python-based interface, the system can be rendered more economical and broadly accessible.  This move not only mitigates the constraints of MATLAB but also corresponds with the overarching objectives of sustainable and open-source technology solutions in renewable energy management.

# Materials

The Grub biogas facility functions as the principal experimental location for this study, supplying critical operational data for model validation and system evaluation. This agricultural biogas facility in Bavaria, Germany, is among the region's most technologically sophisticated installations, comprising a 75 kW gas engine combined heat and power (CHP) unit, a 1206 m³ concrete-covered digester, and a fermentation residue storage capacity of 2714 m³. The facility functions with semi-automated control via Siemens Programmable Logic Controllers (PLC), which oversee and document essential characteristics such as temperature, pressure, and gas flow rates. The plant employs various organic resources for substrate feeding, predominantly including slurry, corn, cattle and sheep dung, grass silage, and fodder leftovers in different ratios. The operational data gathered from this plant encompasses several periods, documenting both steady-state conditions and dynamic reactions to feeding modifications, thereby offering a thorough dataset for the development and validation of mathematical models of the anaerobic digestion process. This empirical data is especially significant as it illustrates the practical difficulties encountered in commercial biogas production, encompassing fluctuations in substrate quality, environmental factors, and operational limitations.

Figure 3.1: Grub  biogas plant

# Methodology

The initial Matlab GUI for adaptable biogas production was developed as a specialized scientific instrument aimed at system model estimate and feedback control for demand-driven biogas generation. The interface was systematically arranged into a sequential workflow that directed users through data preprocessing, model estimates, and control system design. The GUI predominantly focused on two estimating techniques - PT1 approximation and the Time percentage method - hence constraining users' analytical ability in evaluating various modeling strategies.

The interface presented visualizations of biogas production rates and substrate feeding rates, incorporating filtering functionalities to diminish measurement noise using discrete low-pass and smoothing filters. Principal visualizations encompassed time-series graphs for step responses, filtered measurement data, and model estimations, facilitating users in discerning the correlation between substrate feeding and resultant biogas generation. The system enabled users to enter substrate combination data (comprising maize, manure, grass silage, and fodder wastes) and process flow rates recorded by the biogas plant's PLC system.

Notwithstanding its functionality, the Matlab implementation exhibited several significant limitations: its constrained modeling approach provided only two estimation methods instead of the comprehensive six methods (PT1-approximation, time percentage method, turning tangent method, the sum of time constants method, PT1-estimator, and PT2-estimator) delineated in the research. This limitation diminished users' capacity to ascertain the most suitable model for certain biogas plant settings. The system was hindered by an inflexible workflow structure that complicated comparison analysis, while reliance on Matlab licenses imposed financial constraints on implementation at smaller biogas facilities. The interface was deficient in contemporary user experience elements, providing minimal customization possibilities and necessitating an extensive understanding of control theory for efficient functionality.

## Approach to Conversion

Using a mixed approach direct translating fundamental algorithms with strategic redesign of the interface and implementation structure the Matlab GUI for flexible biogas generation was converted to Python.  Although the mathematical underpinnings and data analysis techniques from the original Matlab implementation were maintained, the Python version was developed with improved user interface and expanded capability.

The translation process began with a thorough examination of the original Matlab code to identify key components such as data loading, preprocessing filters, model estimation algorithms, and control system computations. While the implementation technique took advantage of Python's strengths in modern GUI programming and data analysis, essential mathematics and engineering concepts were directly translated to maintain scientific integrity.

The main GUI framework used in the Python implementation is Tkinter, augmented with specific widgets such as tkcalendar's DateEntry for enhanced date selection and tkt.notebook for tabbed navigation. The code for scientific computing depends on accepted Python libraries that parallel Matlab's capability, notably NumPy for numerical operations, pandas for data manipulation, SciPy for signal processing (especially the Butterworth filter implementation), and Matplotlib for visualization. While leveraging Python's more easily available programming paradigm, this mix offers similar analytical power.

Expanding the model estimation capabilities from the two methods (PT1-approximation and Time percentage method) from the original Matlab GUI to include all six estimation approaches described in the research: PT1-estimator, PT2-estimator, Turning tangent method, Sum of time constants method, PT1-estimator, and PT2-estimator was a major redesign aspect. This improvement lets users concurrently evaluate several modeling techniques, therefore offering a complete analysis tool for the optimization of biogas plants.

Designed with enhanced user experience in mind, the interface was rebuilt using a tabbed interface guiding users progressively from data loading to preprocessing, model estimation, control system design, and generating feeding schedules. Beyond what was feasible in the initial Matlab implementation, interactive elements such as sliders for data selection and responsive visualizations offer instantaneous feedback, hence improving the exploratory analytical capacities.

Careful attention was paid to guarantee computational correctness while enhancing usability throughout the conversion process, producing a more accessible but similarly strong tool for flexible biogas production modeling and control.

## Load Data Tab

### Overview

Effective loading and visualization of biogas production and substrate-feeding data from CSV files is made possible by the "Load Data" tab in the Python GUI. This procedure guarantees that the data is organized and available for the next preparation and analysis, therefore complementing approaches developed in relevant research.

4.2.2 Data Acquisition Process

The system focuses on acquiring two primary datasets:

- Biogas Production Rate Data: Extracted from "Gas flow rate Fermenter - current.csv"(Table 1).

- Substrate Feeding Rate Data: Extracted from "Substrate feeding Fermenter - today.csv"(Table 2).

Python's pandas package automatically searches and reads these from the specified directory these files are from. The timestamps are transformed to date time format to allow correct time-series analysis. Concurrent concatenation of the data frames guarantees data continuity.

Table 1 Gas flow rate Fermenter - current

| Time Stamp | Value Num | Validity |
| --- | --- | --- |
| 06.10.2023 00:00:37 | 117.1 | 1 |
| 06.10.2023 00:02:37 | 113.9 | 1 |
| … | … | … |
| 06.10.2023 23:59:28 | 90.8 | 1 |

Table 2 Substrate feeding fermenter - today

| Time Stamp | Value Num | Validity |
| --- | --- | --- |
| 06.10.2023 00:00:37 | 0 | 1 |
| 06.10.2023 00:02:37 | 0 | 1 |
| … | … | … |
| 06.10.2023 23:59:28 | 0 | 1 |

4.2.3 Visualization of Step Graphs

The GUI supports visualizing step-down and step-up behaviours in the biogas production process. The visualization approach includes:

- The plot_step_graph() function creates step graphs depicting biogas production and substrate feeding over time.

- Two graphs were plotted

- Step Downwards (Figure. 1) examines decreasing production and feeding trends.

- Step Upwards (Figure. 2) examines increasing production and feeding trends.

- Matplotlib python library is utilized for plotting, with the primary Y-axis (a blue line) representing the biogas production rate and the secondary Y-axis (a red line) depicting the substrate feeding rate. The x-axis is set up to show hourly timestamps; the last tick mark shows the month and year.

Figure. 4.1 Step downward plot of biogas production rate and substrate feeding before preprocessing

Figure. 4.2 Step upward plot of biogas production rate and substrate feeding before preprocessing

The "Load Data" tab offers an easy-to-use visualizing tool for important step changes in biogas production and substrate feeding and streamlines data collecting. This method  guarantees that the data is ready for preprocessing and next control system analysis, therefore supporting the general aim of improving system modeling and correctness.

## Preprocessing Tab

### Preprocessing of Biogas Production Data

Preprocessing is a key step in ensuring the accuracy and reliability of biogas production data. The fundamental goal is to reduce noise and improve data clarity so that subsequent analysis and management strategies may be built on clean, consistent datasets.

The preprocessing structure is divided into two primary stages. The raw biogas data is initially filtered using a low-pass filtering approach to remove periodic peaks and high-frequency noise. By doing this step, you can be sure that temporary disruptions won't affect the model estimation or control choices. Second, the overall data trend is stabilized by minimizing fluctuations while maintaining crucial dynamic behavior through the use of a moving average smoothing technique. These preprocessing methods guarantee that the ensuing modeling and control procedures run on clean, trustworthy input data and align with accepted practices in control system research.

Both biogas production rate and substrate feeding rate data received the same preprocessing methods, which included Butterworth low-pass filtering and moving average smoothing. The settings of these methods were constantly modified in response to the individual data properties.

#### Low Pass Filtering and Moving Average Smoothing

A fourth-order Butterworth low-pass filter was engineered to address the periodic peaks in biogas production data, which typically arise every 2–3 hours.

The cutoff frequency was determined according to the (Equ. 1) established periodicity:

| cutpfffreq= 12.5×3600 | 1 |
| --- | --- |

This frequency effectively focuses and attenuates periodic peaks, allowing only meaningful and stable trends to pass through the filter. This stage is critical for maintaining data integrity and the control system's reliability.

Following low-pass filtering, the data was flattened, and remaining noise was further reduced by a moving average filter. This approach eliminates tiny swings and stabilises the data trend, resulting in increased clarity. Smoothing is applied with a window size of 5, ensuring that local variances are reduced but not distorting the general trend. This step improves the clarity of the biogas production rate by removing transient variations.

#### Extraction of Feeding Rate

The extraction feeding rate uses a two steps smoothing process to refine data. First, a uniform moving average is applied to low pass filtered data using a predefined smoothing window. Only positive derivatives are kept to prevent artificially negative values by misreading the daily reset at midnight. A post-processing stage then follows to preserve data uniformity since differences in real-world data can magnify noise levels. This entails a threshold-based correcting mechanism (with a threshold value of 0.1) that detects and smooths out considered unnatural rapid decreases in feeding rate. The method iterates over the substrate segment data, comparing adjacent values to ascertain whether they satisfy the threshold; when such cases are found, it substitutes the previous value for the current one, so preserving only real changes in feeding rate in the final dataset.

### Visualization

Figure. 4.3 Downward plot after preprocessed data

Figure. 4.4 Upward plot after preprocessed data

The plot (Figure. 3 & Figure. 4) shows the biogas production rate and substrate feeding rate after preprocessing. The rate of substrate feeding is represented by a solid red line, while the blue dashed line represents the response of biogas production to a sudden change in feeding rate. Although the data still contains a small amount of noise, it is insignificant and has no discernible impact on the measurement.

### GUI Interaction and Control

The Python GUI contains interactive components intended to ease and control the preprocessing steps:

• The Preprocessing Button allows users to apply filtering and smoothing to specified data segments.

• Use sliders to dynamically choose data segments for preprocessing, assuring focussed results.

## Model Estimation Tab

The Model estimation tab uses preprocessed data to try to streamline the analysis and choice of gas production flow rate models. There are two plots for the visualization of the upward and downward steps models. Every part of the models includes graphical representations, correlation measures, and similarity tables in order to evaluate their performances. Six estimating models were implemented: Time Percentage, Pt1_Model, Time Constant Sum, Turning Tangent, Pt1 Estimator, and Pt2 Estimator. The system lets the plant operator select which model to run using a radio button; once a model is chosen, it generates the required changes to the tabular and visual representations. For comparative study, Matplotlib shows similarity metrics in tabular form and facilitates data visualization.

### PT1 - Approximation

The PT1-approximation involves employing a first-order differential equation (Equ. 2) to predict the trajectory of the biogas production rate.

| T⋅ⅆytⅆt+yt=K⋅ut | 2 |
| --- | --- |

In this equation, T signifies the time constant of the system, and K indicates the system gain. The system gain K is determined by the ratio of the change in output to the change in input as in the (Algorithm1, Line 1).

The variation in biogas production rate before and following the step process is denoted as Δy, and the substrate feeding rate is later referred to as Δu. For a step response, whereΔu=1, we have K=Δy. Given that Δy=y∞-y0, and assuming an initial value y0=0, it follows that K=y∞. The time constant T=T63-T0.

Where T0=0 is the starting time of the step process. In this case, the T63 is determined by identifying the point at which the output reaches 63% of its final value. This equation encapsulates the essential dynamic behaviour of the system, highlighting its response time and steady-state characteristics.

An important feature of PT1 systems is their typical response to step inputs, where the output attains about 63% of its final value after one time constant. This percentage is not randomly selected; it arises from the mathematical resolution of the PT1 differential equation.

Assuming that the input variable u(t) is a unit step function, the solution will be:

yt=K⋅1-ⅇ-tT

Due to the unit step function where Δu=1, we have  K=Δy=y∞-y0. Since y0=0, consequently K=y∞. The previous equation may then be rewritten as:

yt=y∞⋅1-ⅇ-tT

For t=T

yT=y∞⋅1-ⅇ-1
we obtain:

yT=y∞1-ⅇ-1≈0⋅63y∞

which approximates to about 63% of the final value y∞. This property offers an effective approach for determining the system's time constant using experimental data.

Once the fundamental parameters have been established, the subsequent step is to develop the system's transfer function. At the outset, the calculated K and T values are employed to construct a continuous transfer function. The function is subsequently converted to a discrete form to facilitate digital implementation, with a sampling time of 120 seconds. The dynamic nature of the system is preserved while the model is effectively utilized with discrete measurement data through the implementation of the conversion process through signal.cont2discrete.

The continuous transfer function was implemented in Python using the control systems representation format, where num = [pt1['K']] indicates the numerator coefficient K and den = [pt1['T'], 1] represents the denominator polynomial  T∙S+1. This method directly converts the mathematical representation of the first-order system to a programmable format.

The method uses the SciPy Signal package to convert discrete transfer functions. Specifically, signal.cont2discrete((num, den), pt1['Ta'], method='zoh') converts the continuous transfer function to its discrete equivalent using the zero-order hold (ZOH) method and a Ta sampling period. This transformation is consistent with the method outlined in algorithm 1, lines 3 and 4 for converting G(s) to Gd(z).

The final stage is to simulate the system response using signals.dlsim((num_d, den_d, pt1['Ta']), t_input, t), which processes the input signal using the discrete transfer function to get the output response. This function internally implements the difference equations that define the discrete-time system, therefore completing the transformation from the continuous s-Laplace domain to the discrete time domain. The Scipy signal library manages the mathematical complexity of this conversion, using suitable difference equations based on the system’s transfer function coefficients extracted from the discrete model.

The implementation identifies and manages both step-down and step-up responses within the system. This adaptability is essential as actual biogas production systems may encounter fluctuations in substrate feeding rates. The system modifies its calculations according to the direction of the step change, guaranteeing precise modelling irrespective of whether the system is increasing or decreasing.

Through the update_model_down_plot_pt1 and update_model_up_plot_pt1 functions, the implementation includes thorough visualization capabilities that visualize the downward (Figure. 5) and upward (Figure. 6) plots. These features produce thorough plots displaying the actual biogas production rate, model predictions, and substrate feeding rate. Easy visual assessment of model performance is made possible by the various line styles and colours used in the plots to distinctly separate measured data from model predictions.

Figure. 4.5 Step downwards visualization of the PT1-approximation method

Figure. 4.6 Step upwards visualization of the PT1-approximation method

The successful implementation of the PT1 approximation depends on fulfilling certain system criteria. The system should mostly exhibit first-order behaviour, with step changes in the input being precisely described. The system dynamics should be collected at an appropriate sample rate (120 seconds in our instance). Furthermore, the signal-to-noise ratio should be sufficient to make exact parameter estimates.

The PT1 approximation is a method that is both accessible and powerful for modelling biogas production systems. The robust modelling tool is the result of the successful integration of theoretical understanding with practical considerations. Although the method's mathematics may be intricate, the implementation is made comprehensible by the clear code structure and comprehensive error handling. The model's performance is easily validated and comprehended through the visual and numerical outputs, rendering it a valuable resource for system optimization and analysis.

| Algorithm 1: PT1-approximation Model |  |
| --- | --- |
|  | Input:Output yt, Input ut, step time t_step |
| 1. | Calculate process gain: k=flowratemax-flowrateminfeedmax-feedmin |
| 2. | Find time constant T63: Critical value=0.63 For step up: ytarget=yinitial+0.63×yfinal-yinitial For step down: ytarget=yinitial-0.63×yinitial-yfinal T=time when ytreaches ytarget-tstep |
| 3. | Continous transfer function: Gc(s)=K(T∙s+1) |
| 4. | Convert to discrete transfer function with sampling time Ta=120s: Gdz=c2d(Gcs, Ta) |
| 5. | Simulate sytem: output=dlsim(Gdz, input, time) |
| 6. | Add offset to output: If stepdown: yd=output+flowratemax stepup: yd=output+flowratemin |

### Time Percentage Method

The time percentage method was employed to model and examine the dynamic relationship between substrate feeding rate and biogas production in anaerobic digestion systems. This method utilizes a second-order differential equation(3) approach, providing superior modeling flexibility relative to basic first-order approximations. The fundamental principle of the method is predicated on the subsequent differential equation:

| TA⋅TB⋅ⅆ2ytⅆt2+TA+TB⋅ⅆytⅆt+yt=K⋅ut | 3 |
| --- | --- |

TA and TB represent the time constants, K is the static gain, y(t) is the system output, and ut is the system input. The process starts with careful data preparation of the substrate feeding rate and biogas-generating rate signals. The step change index is found, and the first and final values are ascertained depending on the step direction; hence, minimum and maximum values are set for inputs and outputs.

Then, by computing the goal value, which represents 72% of the step change, characteristic points are found at which the output meets this value, that is, T1. The second characteristic time point T2=0.2847×T1. The t1 is computed along with the ratio ​y2y∞ where the y2 is the output at the time T2. This ratio is used to look up the coefficients TAT1​ and TBT1​ from the reference table, enabling the calculation of time constants TA,TB​.

Creating the continuous transfer function components using equation. (4), transforming to discrete form using a zero-order hold approach (Zoh) with two sampling periods (120 seconds and 2 hours), and extracting coefficients for the discrete transfer function in the form equation. (5) is the essence of transfer function construction. Model validation is lastly calculated by simulating the system response with the trained model and computing performance measures to evaluate model accuracy.

| Gc(s)=kTA⋅s+1⋅TB⋅S+1 | 4 |
| --- | --- |

| Gdz=fGcS,Ts= e.z+fz2+c.z+d | 5 |
| --- | --- |

The implementation uses Numpy and Scipy libraries to handle numerical operations and transfer function manipulations. The gain calculation is performed using the Equation K (Algorithm 2, line 1). Characteristic points are determined by calculating the goal value according to the step direction. At the same time, the time to achieve 72% of the change (T1) is ascertained by index matching and timestamp conversion.

To calculate time constants, the algorithm uses the ratio of characteristic points from the step response. The normalised output value at the first characteristic point is the ratio of the difference from final value to the entire step magnitude. The normalised time constant row in Table 3 is indexed by this ratio. The program finds equivalent coefficients by searching the reference table for the closest match. After finding the matching row, multiplying the tabulated ratio values by the observed time T1 yields the actual time constants TA and TB, scaling the normalised constants to the system's dynamics.

Table 3 Time constants for the time percentage method

| y2∕y∞ | TA∕t1 | TBt1 |
| --- | --- | --- |
| 0.300 | 0.7950 | 0.000875 |
| …. | …. | …. |
| 0.1610 | 0.4056 | 0.3902 |

Transfer functions are formulated by integrating the distinct components of the second-order system. The components of the continuous transfer function for the PT2 element are mathematically integrated using convolution operations to achieve the comprehensive system representation. This continuous transfer function is then converted to its discrete-time equivalent using the zero-order hold (Zoh) method with a predefined sampling period. The discretised transfer function produces the required coefficients that define the system in the z-domain. The coefficients are extracted to create the difference equation, which establishes a recursive computational model appropriate for digital implementation and simulation of the system's dynamic behaviour.

Figure: 7 and Figure. 8 show the visualization of the time percentage method, illustrating the model's performance for upward and downward step responses respectively.

Figure: 4.7 Step downwards visualization of the Time Percentage method

Figure. 4.8 Step upwards visualization of the Time Percentage method

| Algorithm 2: Time percentage method |  |
| --- | --- |
| Input:flowrate, feed, stepindex, feedmax, feedmin, flowratemax, flowratemin, timestamps, step_direction |  |
| 1 | Calculate process gain: k=(flowratemax-flowratemin)(feedmax-feedmin) |
| 2 | Find first characteristic point T72%: For step up: ytarget=yinitial+0.72×(yfinal-yinitial) For step down: ytarget=yinitial-0.72×(yinitial-yfinal) T1=time when ytr eaches ytarget T72%=T1 |
| 3 | Calculate second characteristic point: T2=0.2847×T1 y2=y(tstep+T2) |
| 4 | Calculate normalized ratio for lookup: For step up: y2ratio=y2-yinitialyfinal-yinitial For step down: y2ratio=(y2-yfinal)(yinitil-yfinal) |
| 5 | Find TA ratioand TB ratiofrom lookup table using y2ratio |
| 6 | Calculate time constants: TA=TA ratio×T1 TB=TBratio×T1 |
| 7 | Continous transfer function: Gcs=K(TA∙s+1)×(TB∙s+1) |
| 8 | Discretize with sampling time Ta: Gdz=e∙z+f(z2+c∙z+d) |
| 9 | Corresponding difference equation:yk+c∙yk-1+d∙yk-2=e∙uk-1+f∙uk-2 Output:K,TA,TB, e, f, c, d |

### Sum of Time Constants Method

The Sum of Time Constants Method is a graphical and computational technique employed to simulate higher-order dynamic systems by modelling them as a sequence of first-order systems. The method simplifies complex dynamic responses by summing individual time constants, making estimating and modelling system behaviour easier.

The process starts by validating the data through the Validate inputs and then calculating the gain K as the ratio between the output and input variables. The process calculates the system gain K (Algorithm 3 line 1) It represents the steady state behavior of the system. The inflection point methodology is a key component of this identifying strategy. The method finds an inflection point (ind_wp) in the step response curve and computes its slope (steigung_wp).

The core of this identification technique is the inflection point methodology [57]. The central point detection is implemented in the find_inflection_point, which functions using numerical gradient calculations. Using the first and second derivatives of the response curve allows the method to identify the inflection point whereby the second derivate approaches zero. By now, the tangent slope is extracted from the first derivative, allowing the construction of the linear approximation to find important temporal markers.

Based on the upward and downward step direction, the method finds the delay time (Tu) and the rise time (Tg) indices by identifying the tangent line intersecting the initial and final steady-state values. The precise time values are computed by converting time differences converted to seconds with the convert_timedelta_to_seconds function to guarantee compatibility with many time formats.

The algorithm then finds a specific reference point at 57.5% of the overall step change (zt['wert_fuer_n_2'] = 0.575) and calculates when the system reaction exceeds this threshold. The overall time constant sum (T_sum) is determined by the time between step initiation and this crossing point. The system order (n) is estimated using the following formula in algorithm 3, line 4.

The individual time constant (T) is then calculated by dividing the total time constant sum by the system order using formula in algorithm 3, line6: T=T_summen

Using the Python control library, the method creates a transfer function model. It starts with a first-order transfer function with the calculated gain and time constant: G= K(Ts+1)

The method then adds (n-1) identical first-order systems in series to represent the complete nth-order system behaviour: G=K((Ts+1)n)

Finally, the approach discretizes the transfer function with a sampling time of 120 seconds and simulates the system's reaction to the actual input signal to confirm the model fit. The simulation modifies the baseline based on the step direction (up or down) to achieve proper alignment with the original data.

This approach allows for accurate modelling of complex dynamic systems while also providing meaningful parameters (gain, time constants, and system order) that characterise the system's response properties, all of which are implemented with numpy for numerical operations and the control library for transfer function manipulation and simulation.

Figure 4.9: Step downwards visualization of the Sum of Time Constants Method

Figure 4.10: Step upwards visualization of the Sum of Time Constants Method

| Algorithm 3: Sum of time constants |  |
| --- | --- |
| Input: |  |
| 1 | Calculate process gain: k=(flowratemax-flowratemin)(feedmax-feedmin) |
| 2 | Tangent line at inflection point: ytangentt=slope*t+offset Where: Slope=derivative at inflection point offset= yinflection-slope*tinflection |
| 3 | Find delay time Tuand rise time Tg: Tu=time from step to tangent'sintersection with initial value Tg=time from Tu to tangent'sintersection with final value |
| 4 | Calculate system order: n=round(TuTg*10+1) |
| 5 | Find time constant sum at 57.5% of step change: Tsum=time to reach 57.5% of final value-t_step |
| 6 | Calculate individual time constant: T=T_sumn |
| 7 | Continous transfer function: Gs=K((T∙s+1)n) |
| 8 | Expanded from of transfer function: Gs=K((T∙s+1)*(T∙s+1)*…*(T∙s+1)) (n identical first ordre terms) |
| 9 | Discretize with sampling time Ta: G(z) Output:K,n,Tu,Tg |

### Turning Tangent Method

Using inflection point analysis, the turning tangent method function offers a comprehensive method of dynamic system identification. This approach uses the geometrical features of step response curves to create estimates of second-order transfer function for process dynamics, therefore offering a useful framework for control system design.

The process starts with the determination of the static gain parameter (K), defined as the ratio between the output range (flowrate_max- flowarte_min⁡) and input range (feed_max- feed_min). This proves, in steady-state settings, the basic amplitude relationship between system input and output. Create a temporal reference for the next computations; the program then finds the step start point, ind_sprung.

Determining the inflection point (ind_WP) and matching slope (steigung_WP), which define the fastest rate of change in the step response, is fundamental to the turning tangent approach. The second derivative of the response curve equals zero at the inflection point, therefore signifying a concavity transition [57]. By now, the method creates a tangent line with the equ. (6)

| y(t) = steigung_wp × t + offset | 6 |
| --- | --- |

where the offset is computed to guarantee that the tangent line intersects the inflection point. The tangent line illustrates the asymptotic behaviour of the system's rate of change and functions as an essential geometric component for deriving temporal parameters.

By finding where the tangent line crosses the first and final steady-state values, the technique finds two main time parameters. While the rising time (Tg) is found as the first point where the tangent goes below the final steady-state value (flowrate_min), the delay time (Tu) corresponds to the last point where the tangent surpasses the initial steady-state value (flowrate_max). For rising steps, the reverse reasoning holds. Time differential computations translate these temporal markers into seconds thereby guaranteeing conformity with technical units.

Based on theoretical connections detailed in the turning tangent approach generates three different time constant estimates [58]:

- T_Twp: Time constant derived from the inflection point time (Twp) with a 1:1 relationship

- T_Tu: Time constant derived from the delay time (Tu) with a 1:0.282 relationship

- T_Tg: Time constant derived from the rise time (Tg) with a 1:2.718 relationship

The analytical solution of second-order systems with equal time constants forms the basis of these connections, where the rising characteristics, inflection point, and delay follow particular mathematical correlations. The technique also averages these three estimations to increase robustness and determine a mean time constant (T_mean).

The method generates a second-order transfer function with the same time constants in the form: for any time constant estimate:

G(s) = K(Ts+1)²

where T stands for the appropriate time constant T_Twp, T_Tu, T_Tg, or T_mean). Common in process control applications, this form correlates with the process having two identical first-order elements in series. Using the zero-order hold (ZOH) approach with a defined sample time (Ta), the technique converts continuous-time transfer functions from scipy.signal library to discrete-time counterparts.

The dlsim function then computes the response to the real input signal modified by the suitable steady-state value, hence simulating the discrete-time models. Unlike straightforward step response simulation, this method guarantees correct reproduction of system behaviour under different input conditions. Offset to match the original process levels, the final simulation results offer a direct comparison between model predictions and measured data.

Reflecting the empirical finding that the rise time usually provides the most consistent basis for model approximation in practical applications, the technique specifies the Tg-based model (yd_Tg) as the main output. This method conforms with Isermann's suggestions for process identification, which state that robust performance throughout a broad spectrum of industrial processes is provided by tangent-based parameters generated from observable response characteristics.

By means of this methodical development from geometric analysis to transfer function modeling and simulation, the turning tangent technique offers a useful framework for characterizing dynamic systems without involving sophisticated frequency-domain testing or specific identification tools. For industrial uses where simplicity, dependability, and physical interpretability are fundamental factors in control system design, this makes it more beneficial.

Figure 4.11. Step downwards visualization of the turning tangent Method

Figure 4.12. Step upwards visualization of the turning tangent Method

| Algorithm 4: Turning tangent method |  |
| --- | --- |
| Input:flowrate, feed,indspring, feedmax, feedmin, flowartemax, flowaretmin, indwp, steigungwp, step_dir |  |
| 1. | Initialize empty dictionary wt for stroing results |
| 2. | Calculate k=(flowratemax-flowratemin)(feedmax-feedmin) |
| 3. | Calculate Wendetangente Inflection TangentLine indexwp= indwp slopewp= steigungwp valuewp= flowrate.interpysmoothindexwp offset = valuewp- slopewp× indexwp tangent_line(t) = slope_wp × t + offset |
| 4. | Determine Tu and Tg Based on Step Direction For stepdir= down: indTu= last index where: tangentlinet> flowratemax indTg= first index where: tangentlinet< flowratemin For stepdir= up: indTu= last index where: tangentlinet< flowratemin ind_Tg = first index where: tangent_line(t) > flowrate_max |
| 5. | Calculate Time Constants Tu = flowrate.interpdtindTu- flowrate.interpdtindsprung Tg = flowrate.interpdtindTg- flowrate.interpdtindTu Twp = flowrate.interp_dt(index_wp) - flowrate.interp_dt(ind_sprung) |
| 6. | Calculate System Time Constant T Using Model Parameters From reference table: TuT= 0.282 TgT= 2.718 TwpT= 1 Therefore: TTwp= Twp1 TTu= Tu0.282 TTg= Tg2.718 T_mean = (T_Twp + T_Tu + T_Tg) / 3 |
| 7. | Continuous Transfer Function For second-order system n = 2: Gs= KT×s+12 Using different T estimates: GTwps= KTTwp×s+12 GTus= KTTu×s+12 GTgs= KTTg×s+12 G_Tmean(s) = K / ((T_mean×s+1)²) |
| 8. | Continuous-Discrete Conversion Ta = sampling time Gdz= c2dGs, Ta Using the Zero-Order Hold (ZOH) method |
| 9. | Calculate Step Response For stepdir= down: ydt= lsimGd, feed.interpdiffsubstratefeed- feedmax, t+ flowratemax For stepdir= up: ydt= lsimGd, feed.interpdiffsubstratefeed- feedmin, t+ flowratemin |
| 10. | Output: wt (structure containing transfer functions and step response) |

### PT1-Estimator

The PT1-Estimator Method determines a first-order transfer function model through the application of least squares estimation to input-output time series data.

The PT1 model is represented by the first-order differential equation:

T⋅ⅆytⅆt+yt=K⋅ut

This can be rewritten in the discrete domain as:

yk+1=a∙yk+b∙u(k)

Where:

- y(k) is the system output (biogas production rate)

- u(k) is the system input (feeding rate)

- a and b are model parameters related to gain and time constant.

Starting with matrix setup where we build the least squares problem matrices the PT1-Estimator process While vector b comprises the ensuring output values y(2) through y(n), matrix A is generated by aggregating columns of past output values y(1) through y(n-1) and input values u(1) through u(n-1).  This layout shows the time-series connection of the system.  (remove we’s from code )

Solving the least squares problem then allows one to estimate parameters a and b that minimise the prediction error between the model and real system behaviour. With scipy.sparse.linalg.lsqr, the implementation effectively computes these parameters even in big datasets.

Following parameter definition, a discrete transfer function is built in the manner G_d(z) = b/(1-az^(-1). This is carried done using control.tf([b],[1, -a],sampling_time), therefore producing a mathematical representation of the input-output link of the system.

By means of the identical input sequence applied in the original data and comparing the model's output against measured values, system response simulation validates the found model at last.  This validation phase guarantees the accuracy and fit of the model for control or prediction uses.

The PT1-Estimator method has a computational simplicity relative to the Time Percentage Method as one advantage. It does not depend on characteristic points or lookup tables and calls for fewer computations.

Systems with dominating first-order behaviour, situations when computing economy is sought, and applications with limited data or noisy observations will find the PT1-Estimator appropriate. Higher-order dynamics could thus not be as well captured as the Time Percentage Method, hence model errors could result when the system shows notable second-order characteristics. When deciding the modelling technique to use for biogas generation systems, this restriction becomes a significant factor since the suitable technique should be selected depending on the expected complexity of the underlying process dynamics.

The continuous-time transfer function matching the determined model is:

Gs= KTs+1

T is the temporal constant; K, derived from the discrete parameters a and b, is the static gain.

By means of data-driven approaches, the PT1-Estimator method offers a direct identification of the model parameters, therefore removing the requirement for either graphical or manual parameter selection.

| Algorithm: 5 PT1-estimator |  |
| --- | --- |
| Input: flowrate, feed, feed_boundaries, flowrate_boundaries, step_direction |  |
| 1. | Setup system identification matrices Use current values to predict next values: yk+1= a*yk+ b*uk A = flowrate0:end-1, feed0:end-1 (Matrix with flowrate and feed data) b = flowrate[1:end] (Vector with next timestep flowrate values) |
| 2. | Solve linear system using least squares Find parameters x = a, bthat minimize A*x - b2 x = least_squares_solve(A, b) |
| 3. | Create discrete transfer function Gz=bz-a sampletime= 120 transfer_function = discrete_tf(numerator=[x[1]], denominator=[1, -x[0]], sample_time) |
| 4. | Calculate system response based on step direction testtime= timevector0, sampletime* lengthflowrate Calculate response for step down inputsignal= feed - feedmax response = simulatesystemtransferfunction, testtime, inputsignal result.response = response + flowrate_max |
| 5. | Calculate response for step up inputsignal= feed - feedmin response = simulatesystemtransferfunction, testtime, inputsignal result.response = response + flowrate_min |
| 6. | Return estimated model and its parameters result.parameters = x result.transferfunction= transferfunction result.simulation_time = test_time |

### PT2-Estimator

Using a second-order discrete-time model directly built from input-output data, the PT2-Estimator approach approximates system dynamics. Unlike graphical techniques, this method finds the coefficients of a second-order differential equation via least squares parameter estimation.

A second-order difference equation in the form discussed models the system:

yk=a1∙y(k-1)+a2∙y(k-2)+b1∙u(k-1)+b2∙u(k-2)

The process starts with matrix formulation, in which input and output data build a matrix form system of equations. Whereas vector b comprises the matching output values to be anticipated, matrix A has rows of historical output and input values. Solving the least squares problem then helps to estimate the parameters by obtaining the parameter vector [a1, a2, b1,b2] that reduces the squared error between the projected and real outputs. The discrete transfer function is then built from these found parameters, producing a rational function G_d(z) with a numerator comprising b-parameters and a denominator comprising a-parameters, all with suitable delay terms (Z-1, Z-2). At last, model validation is carried out by computing the response of the model to the input sequence and then verifying the accuracy of the identification process by means of a measured output comparison.

Applications of biogas plant modelling benefit much from the PT2-Estimator approach. By so faithfully capturing oscillatory dynamics and second-order effects common in complex biological systems, it provides better model fidelity than PT1 models. One of its main advantages is direct parameter identification, therefore removing the necessity for certain transient response tests that graphical techniques usually demand.  By use of its least squares formulation, the method shows resilience in managing both input and output noise, hence producing stronger parameter estimate than conventional graphical methods.  When producing biogas, when dynamics cannot be sufficiently explained by simpler first-order models, the PT2-Estimator is especially appropriate.  Its second-order structure effectively models the complex behaviours often observed in biogas production plants by faithfully representing both fast and slow dynamic components that define biological processes.

The discrete model has an equivalent continuous transfer function of the form:

Gs=K(Tzs+1)(Ta1s+1)(Ta2s+1)

Thus, the PT2-Estimator offers a data-driven method to derive a second-order model of biogas generation without depending on particular step response tests or distinctive point measurements as necessary in the Time Percentage Method.

| Algorithm: 6 PT2-estimator |  |
| --- | --- |
| Input:(flowrate, feed, feed_max, feed_min, flowrate_max, flowrate_min, step_dir) |  |
| 1. | Setup system identification matrices for second-order model Form: yk= a1*yk-1+ a2*yk-2+ b1*uk-1+ b2*uk-2 A = [ flowrate1:-1, //yk-1 flowrate0:-2, //yk-2 feed1:-1, // uk-1 feed[0:-2] // u(k-2) ] b = flowrate[2:] // y(k) |
| 2. | Solve linear system using least squares Find parameters x = a1, a2, b1, b2that minimize A*x - b2 x = least_squares_solve(A, b) |
| 3. | Setup for discrete-time domain samplingtime= 120 time_vector = generate_time_vector(0, sampling_time, length(flowrate)) |
| 4. | Create discrete transfer function for PT2 model Gz=b1*z + b2z2- a1*z - a2 numerator = x2, x3 denominator = [1, -x[0], -x[1]] |
| 5. | Calculate system response based on step direction Calculate response for step down inputsignal= feed - feedmin response = simulate_discrete_system(numerator, denominator, samplingtime, inputsignal) result.response = response + flowrate_min |
| 6. | Return estimated model and its parameters result.parameters = x result.numerator = numerator result.denominator = denominator result.simulation_time = time_vector |

## Control system

After an extensive evaluation of six distinct system models, the time percentage method was chosen for the step upward (second-order differential equation) model. The control system was engineered to position all closed-loop poles at the origin, employing a discrete transfer function derived from the time percentage method during model estimation. This feedback control system implemented a deadbeat control strategy with anti-windup compensation to address control signal saturation. The system design followed to the framework depicted in Fig. 7, integrating a prefilter gain F, state feedback gain K, and saturation limits to regulate the substrate feeding rate within operational parameters.

### System Identification and Modelling

The system starts with a discrete transfer function derived from the selected model in the model estimation tab, which is the time percentage method. The coefficients of the transfer function were extracted from the time percentage analysis using the equation (5):

Gdz= e∙z+fz2+c∙z+d

This transfer function represents the dynamic relationship between input (substrate feeding rate) and output (Biogas production flow). The transfer function is derived from the preprocessing result and transformed into a state space representation for the controller design.

### State Space Representation

The system is converted to state space form with the following equations (Algorithm line 2):

xk+1=Axk+Buk

yk=Cxk

Where xk is the state vector at time step k, uk is the control input, yk the output and A, B and C are the state space matrices derived from the transfer function coefficients:

A=a1a2a3a4

B=b1b2

C=[c1c2]

The transformation to state space is executed using control system library functions to guarantee mathematical precision.

### Controller Design

A deadbeat controller is used to attain rapid response with a low settling time for the biogas production system.

The deadbeat controller implementation through the design_deadbeat_controler() function works to place all closed-loop poles at the origin (zero) using Ackermann's formula to achieve the fastest possible response without overshooting. In the event of numerical problems, the implementation uses near zero poles or manual calculation as fallback approaches. The control law uses state feedback:

uk=Frk-Kxk

Where  F is the prefilter gain, rk is the reference input (setpoint) and K is the state feedback gain matrix.

### Anti-Windup and Saturation Implementation

To reflect the biogas plant's operational constraints, the control system included a saturation block that limited the substrate feeding rate to set minimum and maximum values.

The saturation was implemented as:

uboundedk=sat(uunboundedk,umin,umax)

Where:

sat is the saturation function

umin= feedmin-feedinitial

umax=feedmax-feedinitial

This saturation mechanism avoided excessive control operations while keeping the system stable. The anti-windup approach ensured that the state feedback controller did not develop integral errors while the control signal was saturated.

### Prefilter Design

The Prefilter gain is calculated using:

F=[C(I-A+BK)-1B]-1

This guarantees no steady-state error for step inputs by appropriate scaling of the reference signal.

### Simulation Framework

The control system simulation was run for 7200 seconds (2 hours) over a time period specified by the user. At each simulation step in Figure 4.13, the following procedure was performed: the simulation calculated state feedback from the current state vector identified the prefilter output from the setpoint deviation and computed the unbounded control signal as the difference between these values. This control signal was then passed to the saturation block to ensure that it stayed within the operational parameters for substrate feeding. Using the bounded control signal, the state vector was updated in accordance with the state space equations, and the biogas production rate was estimated by applying the C matrix to the state vector and adding the initial flow rate. The simulation saved time series data for biogas generation and substrate feeding rates, which were displayed on a dual-axis plot that depicted the system's response to setpoint changes and the related control actions. This solution allowed for performance evaluation under various operating situations while remaining within the biogas plant's physical limitations.

Figure 4.13: Feedback control loop for the upward model using state space controller

| Algorithm 4: Feedback Control System for Biogas Production |  |
| --- | --- |
| Input: Initial gas flow rate, Gas flow setpoint, Initial substrate feeding rate, Simulation duration, Feeding rate limits (min and max), Transfer function coefficients (e, f, c, d) |  |
| 1 | System Transformation Extract transfer function coefficients from preprocessing Gdz= e∙z+fz2+c∙z+d |
| 2 | Convert to state space representation: xk+1=Axk+Buk yk=Cxk |
| 3 | Controller Design Design deadbeat controller using Ackermann’s formula Place eigenvalues of (A-BK)at the origin. |
| 4 | Delta set point Delta setpoint=Gas production flow rate Setpoint-Gas production flow rate Initial |
| 5 | Calculate Prefilter gain F=[C(I-A+BK)-1B]-1 |
| 6 | Incorporate these parameters into the feedback control scheme depicted in Figure (4.13) |
| 7 | Output: Simulated and extracted substrate feeding plan |

# Result and Discussion

## Results 1: Model Estimation

In this study, six different model estimation methods were implemented to evaluate their ability to characterize the dynamic behavior of biogas production in response to step changes in substrate feeding. The PT1 approximation, time percentage method, time constant sum, turning tangent, PT1-estimator, and PT2-estimator were among these techniques. The coefficient of determination (R2) was used to evaluate each method's performance after it was applied to the same experimental dataset.

As shown in table (Table 4), Out of the six, the Time Percentage Method (TPM) has the greatest R2 value of 0.9932, making it the most accurate. Accordingly, TPM was able to account for almost 99.32% of the variation in the biogas production rate that was seen after an input that was stepped up. Compared to the turning tangent technique, which performed noticeably worse (R2 = 0.3312), the PT1 approximation and time constant sum method demonstrated somewhat lesser accuracy (R2 = 0.9900 and 0.9889, respectively), indicating that they are not suitable for modeling the dynamics of anaerobic digestion.

Table 4: Evaluation values for the upward model

| Estimation method | R² (%) |
| --- | --- |
| Time percentage method | 0.9932 |
| PT1 approximation | 0.9900 |
| Time constant sum | 0.9889 |
| Turning tangent | 0.3312 |
| PT1-estimator | 0.8243 |
| PT2-estimator | 0.8265 |

The Time Percentage Method (TPM) is based on a second-order differential equation that better describes how biogas production systems change over time, especially when material feeding goes up. Second-order models start with a zero tangent at the origin, which means that changes happen slowly. This is different from first order (PT1) models, which start with a nonzero slope, meaning the model responds immediately to input changes. This property fits biologically with the anaerobic digestion process, in which microbial groups need time to break down new substrates before biogas production starts to rise. This delay is an important part of the system that needs to be correctly recorded for control to work. Because of this, TPM's second-order approximation makes the model for simulating upward changes in biogas output more realistic and reliable.

The application of TPM to the dataset is illustrated in the graph (Figure. 4.8). The red line displays the substrate feeding rate (uk), the yellow dotted line shows the estimated output (ŷk) of the model, and the blue dashed line shows the measured biogas production rate (yk). The upward dynamics employed for model identification are started when a clear step input is applied at around 07/19:00 when the substrate feeding rate rises from about 0.2 t/h to 0.65 t/h.

To develop the second-order transfer function for the upward model, the time constants TA and TB are found using the two points T1 and T2 (Figure. 4.8). When modeled using TPM, the resulting step response closely resembles the behavior of the plant and shows a smooth, progressive growth without overshoot.

The Time Percentage Method works because it is biologically related to the process of anaerobic digestion. When substrate feeding goes up, bacteria need time to digest the extra food before making more methane. The second-order differential equation from the Time Percentage Method, which has a zero tangent at the origin, perfectly describes this cellular delay.

The mathematical structure of the method matches the natural lag phase in microbial activity, which is why it is more accurate (R² = 0.9932) than other models that were tried. It's great for modeling upward changes in biogas systems because it can show both the original adaptation and the subsequent production increase.

The upward model was purposefully used in the simulation, which is significant. The ability to anticipate and manage the rise in biogas production during periods of high demand or price variations is increasingly important in real-world situations. Accurate modeling of upward dynamics is essential for control system design since it is more difficult to simulate because of the nonlinearity and latency in microbial response. Hence, concentrating on the upward model supports the objective of demand-oriented biogas generation while also offering greater insight into adaptable production strategies.

The excellent fit between the anticipated biogas curve and the actual observations confirms the TPM's applicability for simulating upward transitions. The state-space feedback controller created in the thesis's subsequent section is based on the model structure produced by this technique.

## Results 2: Feedback Control Design

The system models have been developed, and the feedback control system will now be tested. This study focuses entirely on the upward model with a state space controller, which was converted to Python from the original MATLAB design.

Figure 5.1 illustrates how the state space controller controls the upward model. The yellow dashed line represents the setpoint of roughly 95 m³/h. The blue line depicts the simulated biogas production flow rate utilizing the state space controller, starting at 83 m³/h and progressively increasing to the setpoint level. The red line shows the substrate feeding rate, which the controller adjusts to reach the target biogas generation.

The Python implementation of the state space controller effectively tackles the major problem of flexible biogas production: attaining predictable output changes while preserving process stability. The simulation results show that the controller can consistently boost biogas output from an initial state to a desired setpoint by altering substrate feeding rates. The controller-generated feeding schedule serves as a practical bridge between theoretical control algorithms and real-world plant functioning. The Python-based variant has a far better user interface than the original MATLAB version. The graphical user interface clearly visualizes both inputs and outputs, allowing operators to monitor the control process intuitively. Furthermore, the Python code links the controller with automatic feeding schedule generation, resulting in a complete solution for flexible biogas production management.

The transfer from MATLAB to Python preserved the mathematical precision of the control algorithms while increasing accessibility and usefulness. Throughout the conversion procedure, special care was taken to ensure the numerical stability of the state space calculations. The final implementation indicates that open-source software platforms may effectively support advanced control strategies for biogas facilities while maintaining performance.

Overall, the simulation results show that the state space controller written in Python is effective at controlling flexible biogas production in the upward direction. The controller successfully converts mathematical models into realistic feeding schedules that can be implemented in real biogas plants to achieve demand-oriented production, demonstrating the feasibility of Python-based control systems for industrial biogas applications.

Figure 5.1: Simulation results of the state space controller for upward model

This graph explains why the state space controller was chosen as the upward model in this investigation. The controller has a smooth response curve with no oscillations and reaches the setpoint rapidly. The biogas production rate gradually increases over time, demonstrating the controller's capacity to manage the complex biological dynamics involved in the anaerobic digestion process. Unlike other controllers, the state space controller does not overshoot the target value, which is essential for maintaining stable biological conditions in the digester.

The Python solution retains all of the fundamental aspects of the original MATLAB-based controller while providing more accessibility and cost-efficiency. Converting the control algorithms to Python eliminated the requirement for proprietary software licensing, resulting in a more accessible platform for biogas plant operators. The Python simulation interface visualizes the control process in a comprehensible manner and makes practical implementation easier by creating precise feeding plans.

Table 5 displays a thorough feeding schedule created from the controller output. The schedule outlines a time-based strategy for feeding substrate mixtures throughout a 24-hour period, with particular amounts for each substrate component (corn, manure, and fodder residues). The feeding schedule starts with a small amount (0.4200 t) at 08:00 and gradually grows to 0.8000 t over numerous intervals, with minor changes during the day. This pattern of substrate addition is directly proportional to the regulated increase in biogas production seen in the simulation graph.

Table 5: Two hourly feeding schedule, the controller output

| Time | Corn [t] | Manure [kg] | Fodder residues [t] | Sum [t] |
| --- | --- | --- | --- | --- |
| 08:00 | 0.0788 | 0.1050 | 0.0262 | 0.4200 |
| 10:00 | 0.1500 | 0.2000 | 0.0500 | 0.8000 |
| 12:00 | 0.1500 | 0.2000 | 0.0500 | 0.8000 |
| 14:00 | 0.1500 | 0.2000 | 0.0500 | 0.8000 |
| 16:00 | 0.1422 | 0.1896 | 0.0474 | 0.7583 |
| 18:00 | 0.1214 | 0.1618 | 0.0405 | 0.6473 |
| 20:00 | 0.1284 | 0.1712 | 0.0428 | 0.6849 |
| 22:00 | 0.1284 | 0.1712 | 0.0428 | 0.6849 |
| … | … | … | … | … |
| 06:00+1d | 0.1284 | 0.1712 | 0.0428 | 0.6849 |

The feeding schedule is intended to be easily implementable by plant operators. Each time segment defines the precise amount of various substrate components to be added to the digester. For example, at 08:00, the operator would apply 0.0788 t of maize, 0.1050 t of manure, and 0.0262 t of fodder residue. The schedule maintains constant feeding patterns for extended periods as needed, easing the operator's task while attaining the essential control objectives.

# Conclusion

This thesis has addressed the fundamental challenge of demand-driven biogas production by designing and implementing an advanced feedback control system focusing on converting control algorithms from MATLAB to Python. The research directly addresses the critical need for flexible energy production systems capable of adapting to changing market needs as part of the transition toward renewable energy infrastructure.

The comparative investigation of six different estimation methods provided important insights into system modeling approaches. The Grey box method was particularly effective, successfully merging theoretical frameworks from White box methodologies with data-driven parameter estimation techniques characteristic of Black box approaches. Following rigorous evaluation using the R² coefficient of determination, the time percentage method based on second-order differential equations was identified as optimal for the upward production model. This method demonstrated superior accuracy when validated against simulation and experimental datasets.

The state space controller designed for the upward model demonstrated remarkable performance measures, including stable, non-oscillating behavior and exact setpoint tracking. This research’s primary contribution is translating complex control theory into workable feeding schedules that biogas plant operators can immediately apply. This successful bridging of theoretical models with practical applications represents a significant advancement in making flexible biogas production accessible to operators who may not have strong backgrounds in control theory.

Beyond simple code transfer, the switch from MATLAB to Python achieved multiple strategic goals, including reduced reliance on proprietary software licenses, increased accessibility for plant operators, and improved user experience via intuitive graphical interfaces. The integration of controller algorithms and automated feeding plan generation has resulted in a comprehensive solution that covers the entire operational workflow of flexible biogas production management.

Despite its results, this study has limitations in accounting for all biogas production variables, such as substrate qualities and environmental circumstances. Future model improvements could include real-time monitoring with sensors and machine learning to help plants adapt to changing conditions.

Future research should focus on creating a unified control system that can handle both increasing and decreasing production demands. This might integrate the state space controller with PI-controller parts and an Anti-Windup scheme, as well as investigate how flexible operation affects digester biology and optimize substrate combinations for increased flexibility.

This study conclusively indicates that flexible biogas production may be achieved using relatively simple control systems implemented on widely available software platforms. The Python-based control system developed here provides biogas plant operators with a feasible path to participate in demand-driven energy markets, possibly increasing both the economic feasibility of their operations and their contribution to grid stability in renewable energy systems. This work contributes to the greater goal of more successfully integrating biogas into the renewable energy environment, so facilitating the global transition to sustainable, adaptable, and resilient energy infrastructure

References

[1]	H. Grassl, “Climate Change Challenges,” Surv Geophys, vol. 32, 4-5, pp. 319–328, 2011, doi: 10.1007/s10712-011-9129-z.

[2]	A. Hassan, S. Z. Ilyas, A. Jalil, and Z. Ullah, “Monetization of the environmental damage caused by fossil fuels,” Environmental science and pollution research international, vol. 28, no. 17, pp. 21204–21211, 2021, doi: 10.1007/s11356-020-12205-w.

[3]	F. Perera, “Pollution from Fossil-Fuel Combustion is the Leading Environmental Threat to Global Pediatric Health and Equity: Solutions Exist,” International journal of environmental research and public health, vol. 15, no. 1, 2017, doi: 10.3390/ijerph15010016.

[4]	F. Martins, C. Felgueiras, M. Smitkova, and N. Caetano, “Analysis of Fossil Fuel Energy Consumption and Environmental Impacts in European Countries,” Energies, vol. 12, no. 6, p. 964, 2019, doi: 10.3390/en12060964.

[5]	L. Cherwoo et al., “Biofuels an alternative to traditional fossil fuels: A comprehensive review,” Sustainable Energy Technologies and Assessments, vol. 60, p. 103503, 2023, doi: 10.1016/j.seta.2023.103503.

[6]	G. Leonzio, “Biogas Produced from Different Feedstocks in Anaerobic Digesters,” in Topics in Mining, Metallurgy and Materials Engineering, Nanotechnology in Oil and Gas Industries, T. A. Saleh, Ed., Cham: Springer International Publishing, 2018, pp. 291–338.

[7]	M. K. Jameel et al., “Biogas: Production, properties, applications, economic and challenges: A review,” Results in Chemistry, vol. 7, p. 101549, 2024, doi: 10.1016/j.rechem.2024.101549.

[8]	P. Weiland, “Biogas production: current state and perspectives,” Applied microbiology and biotechnology, vol. 85, no. 4, pp. 849–860, 2010, doi: 10.1007/s00253-009-2246-7.

[9]	PowerUp – Parts for Gas-engines, What are the Main Components of Biogas Plants? I PowerUP. [Online]. Available: https://​www.powerup.at​/​knowledge/​biogas-​plant/​components/​ (accessed: Feb. 5 2025).

[10]	N. Scarlat, J.-F. Dallemand, and F. Fahl, “Biogas: Developments and perspectives in Europe,” Renewable Energy, vol. 129, pp. 457–472, 2018, doi: 10.1016/j.renene.2018.03.006.

[11]	L. A. Putra, B. Huber, and M. Gaderer, “Real-world application of a discrete feedback control system for flexible biogas production,” Adv. Model. and Simul. in Eng. Sci., vol. 10, no. 1, 2023, doi: 10.1186/s40323-023-00251-1.

[12]	Dr. Phillip M. Feldman, Eight Advantages of Python Over Matlab. [Online]. Available: https://​phillipmfeldman.org​/​Python/​Advantages_​of_​Python_​Over_​Matlab.html (accessed: Feb. 5 2025).

[13]	Ceyhun Ozgur, Valparaiso University, ceyhun.ozgur@valpo.edu Taylor Colliau, Valparaiso University, taylor.colliau@valpo.edu, “MatLab vs. Python vs. R,” 2017.

[14]	R. Schaeffer et al., “Energy sector vulnerability to climate change: A review,” Energy, vol. 38, no. 1, pp. 1–12, 2012, doi: 10.1016/j.energy.2011.11.056.

[15]	J. Houghton, “The science of global warming,” Interdisciplinary Science Reviews, vol. 26, no. 4, pp. 247–257, 2001, doi: 10.1179/isr.2001.26.4.247.

[16]	O. Bashiru, C. Ochem, L. A. Enyejo, H. N. N. Manuel, and T. O. Adeoye, “The crucial role of renewable energy in achieving the sustainable development goals for cleaner energy,” Global J. Eng. Technol. Adv., vol. 19, no. 3, pp. 11–36, 2024, doi: 10.30574/gjeta.2024.19.3.0099.

[17]	I. Sárvári Horváth, M. Tabatabaei, K. Karimi, and R. Kumar, “Recent updates on biogas production - a review,” BRJ, vol. 3, no. 2, pp. 394–402, 2016, doi: 10.18331/BRJ2016.3.2.4.

[18]	K. Obileke, N. Nwokolo, G. Makaka, P. Mukumba, and H. Onyeaka, “Anaerobic digestion: Technology for biogas production as a source of renewable energy—A review,” Energy & Environment, vol. 32, no. 2, pp. 191–225, 2021, doi: 10.1177/0958305X20923117.

[19]	“modeling-approaches-of-biogas-production-3,”

[20]	A. K. Duun-Henriksen et al., “Model identification using stochastic differential equation grey-box models in diabetes,” Journal of diabetes science and technology, vol. 7, no. 2, pp. 431–440, 2013, doi: 10.1177/193229681300700220.

[21]	M. Heiker, M. Kraume, A. Mertins, T. Wawer, and S. Rosenberger, “Biogas Plants in Renewable Energy Systems—A Systematic Review of Modeling Approaches of Biogas Production,” Applied Sciences, vol. 11, no. 8, p. 3361, 2021, doi: 10.3390/app11083361.

[22]	K. Waszkielis, I. Białobrzewski, and K. Bułkowska, “Application of anaerobic digestion model No. 1 for simulating fermentation of maize silage, pig manure, cattle manure and digestate in the full-scale biogas plant,” Fuel, vol. 317, p. 123491, 2022, doi: 10.1016/j.fuel.2022.123491.

[23]	S. Tisocco, S. Weinrich, G. Lyons, M. Wills, X. Zhan, and P. Crosson, “Application of a simplified ADM1 for full-scale anaerobic co-digestion of cattle slurry and grass silage: assessment of input variability,” Front. Environ. Sci. Eng., vol. 18, no. 4, 2023, doi: 10.1007/s11783-024-1810-9.

[24]	M. Ehmer and F. Khan, “A Comparative Study of White Box, Black Box and Grey Box Testing Techniques,” IJACSA, vol. 3, no. 6, 2012, doi: 10.14569/IJACSA.2012.030603.

[25]	F. Amara, K. Agbossou, A. Cardenas, Y. Dubé, and S. Kelouwani, “Comparison and Simulation of Building Thermal Models for Effective Energy Management,” SGRE, vol. 06, no. 04, pp. 95–112, 2015, doi: 10.4236/sgre.2015.64009.

[26]	The Economy, Sustainable Development, and Energy International Conference. Basel Switzerland: MDPI, 2018.

[27]	N. Raeyatdoost, R. Eccleston, and C. Wolf, “Flexible Methane Production Using a Proportional Integral Controller with Simulation‐Based Soft Sensor,” Chem Eng & Technol, vol. 43, no. 1, pp. 75–83, 2020, doi: 10.1002/ceat.201900401.

[28]	E. Mauky, S. Weinrich, H.-J. Nägele, H. F. Jacobi, J. Liebetrau, and M. Nelles, “Model Predictive Control for Demand‐Driven Biogas Production in Full Scale,” Chem Eng & Technol, vol. 39, no. 4, pp. 652–664, 2016, doi: 10.1002/ceat.201500412.

[29]	A. Kovalovszki, M. Alvarado-Morales, I. A. Fotidis, and I. Angelidaki, “A systematic methodology to extend the applicability of a bioconversion model for the simulation of various co-digestion scenarios,” Bioresource technology, vol. 235, pp. 157–166, 2017, doi: 10.1016/j.biortech.2017.03.101.

[30]	F. Haugen, R. Bakke, and B. Lie, “On-off and PI Control of Methane Gas Production of a Pilot Anaerobic Digestion Reactor,” MIC, vol. 34, no. 3, pp. 139–156, 2013, doi: 10.4173/mic.2013.3.4.

[31]	S. Attar and F. Haugen, “Model-based optimal recovery of methane production in an anaerobic digestion reactor,” MIC, vol. 41, no. 2, pp. 121–128, 2020, doi: 10.4173/mic.2020.2.7.

[32]	S. Fawzy, M. Saeed, A. Eladl, and M. El-Saadawi, “Adaptive Control System for Biogas Power Plant Using Model Predictive Control,” Journal of Modern Power Systems and Clean Energy, vol. 9, no. 5, pp. 1193–1204, 2021, doi: 10.35833/MPCE.2019.000170.

[33]	M. Sbarciog, M. Loccufier, and E. Noldus, “The Estimation of Stability Boundaries for an Anaerobic Digestion System,” IFAC Proceedings Volumes, vol. 43, no. 6, pp. 359–364, 2010, doi: 10.3182/20100707-3-BE-2012.0032.

[34]	K. Yetilmezsoy, K. Karakaya, M. Bahramian, S. A. Abdul-Wahab, and B. İ. Goncaloğlu, “Black-, gray-, and white-box modeling of biogas production rate from a real-scale anaerobic sludge digestion system in a biological and advanced biological treatment plant,” Neural Comput & Applic, vol. 33, no. 17, pp. 11043–11066, 2021, doi: 10.1007/s00521-020-05562-7.

[35]	M. H. Perng, “控制系統（二）,”

[36]	O. Bernard, Z. Hadj-Sadok, D. Dochain, A. Genovesi, and J. P. Steyer, “Dynamical model development and parameter identification for an anaerobic wastewater treatment process,” Biotechnology and bioengineering, vol. 75, no. 4, pp. 424–438, 2001, doi: 10.1002/bit.10036.

[37]	D. Dochain and P. Vanrolleghem, “Dynamical Modelling & Estimation in Wastewater Treatment Processes,” Water Intelligence Online, vol. 4, no. 0, 9781780403045-9781780403045, 2015, doi: 10.2166/9781780403045.

[38]	P. V. Denis Dochain, “Dochain Vanrolleghem 2015 dynamical modelling_compressed-1-120,” 2001.

[39]	K. Ogata, “Modern control engineering,” 2010.

[40]	F. Haugen, R. Bakke, and B. Lie, “State Estimation and Model-Based Control of a Pilot Anaerobic Digestion Reactor,” Journal of Control Science and Engineering, vol. 2014, pp. 1–19, 2014, doi: 10.1155/2014/572621.

[41]	Bernard, O., Hadj-Sadok, Z., Dochain, D., Genovesi, A., & Steyer, J. P, “Dynamical model development and parameter identification for an anaerobic wastewater treatment process,” 2001.

[42]	K. P. Burnham and D. R. Anderson, “Multimodel Inference,” Sociological Methods & Research, vol. 33, no. 2, pp. 261–304, 2004, doi: 10.1177/0049124104268644.

[43]	A.-N. Spiess and N. Neumeyer, “An evaluation of R2 as an inadequate measure for nonlinear models in pharmacological and biochemical research: a Monte Carlo approach,” BMC pharmacology, vol. 10, p. 6, 2010, doi: 10.1186/1471-2210-10-6.

[44]	A. Donoso-Bravo, J. Mailier, C. Martin, J. Rodríguez, C. A. Aceves-Lara, and A. Vande Wouwer, “Model selection, identification and validation in anaerobic digestion: a review,” Water research, vol. 45, no. 17, pp. 5347–5364, 2011, doi: 10.1016/j.watres.2011.08.059.

[45]	Lennart Ljung, “Black-box Models from Input-output Measurements,” Oct. 2001.

[46]	T. Chai and R. R. Draxler, “Root mean square error (RMSE) or mean absolute error (MAE)? – Arguments against avoiding RMSE in the literature,” Geosci. Model Dev., vol. 7, no. 3, pp. 1247–1250, 2014, doi: 10.5194/gmd-7-1247-2014.

[47]	G. K. Sadanori Konishi, “Information Criteria and Statistical Modeling,” 2007.

[48]	W. L. Martinez, “Graphical user interfaces,” WIREs Computational Stats, vol. 3, no. 2, pp. 119–133, 2011, doi: 10.1002/wics.150.

[49]	E. L. Hutchins, J. D. Hollan, and D. A. Norman, “Direct Manipulation Interfaces,” Human–Computer Interaction, vol. 1, no. 4, pp. 311–338, 1985, doi: 10.1207/s15327051hci0104_2.

[50]	C. Depcik and D. N. Assanis, “Graphical user interfaces in an engineering educational environment,” Comput. Appl. Eng. Educ., vol. 13, no. 1, pp. 48–59, 2005, doi: 10.1002/cae.20029.

[51]	I. P. Idoko, G. C. Ezeamii, C. Idogho, E. Peter, U. S. Obot, and V. A. Iguoba, “Mathematical modeling and simulations using software like MATLAB, COMSOL and Python,” Magna Sci. Adv. Res. Rev., vol. 12, no. 2, pp. 62–95, 2024, doi: 10.30574/msarr.2024.12.2.0181.

[52]	D. Xue and Y. Chen, Modeling, Analysis and Design of Control Systems in MATLAB and Simulink: WORLD SCIENTIFIC, 2014.

[53]	F. J. Folgado, I. González, and A. J. Calderón, “STUDYING MATLAB APPDESIGNER AS ALTERNATIVE TOOL FOR TEACHING INDUSTRIAL MONITORING/SUPERVISION,” in INTED2022 Proceedings, Online Conference, 2022, pp. 2892–2898.

[54]	“Scalability of matlab,”

[55]	lalit, “python for beginners,”

[56]	“Developing a graphical user interface for viewing and editing behvior tree polices,”

[57]	R. Isermann and M. Münchhof, “Identification of Dynamic Systems,” 2011, doi: 10.1007/978-3-540-78879-9.

[58]	W. Schneider, Praktische Regelungstechnik: Ein Lehr- und Übungsbuch für Nicht-Elektrotechniker ; mit 72 Tabellen und 54 Übungsaufgaben, 3rd ed. Wiesbaden: Vieweg + Teubner, 2008. Accessed: Feb. 27 2025.